How to setup SMB scanning for Kyocera & Copystar

network share folder

1. Open up the web configuration page via the copiers IP address.  Login user name is “Admin” and password is “Admin” and click “Login”.  If the model has the previous generation network controller and only asks for a password it is “admin00”.

2. Go to the “Address Book” then select “Add Contact”

3. Complete the “Contact Information” & “SMB” section

-Host Name is the server name or IP Address of the server

-Port number should be 139 (some 2008 servers use 445)

-Path is the share folder (use “\” to separate sub folders)

-User Name and Password with permission to the share folder (note: if the password is set to change ever 30 or 90 day the end user will have to update this themselves every time their password change.  It is preferred for this reason to use an account that does not have a password expiration).

Test scanning on the copier by choosing “Send” then “Address Book”; select the address book destination and press the start key.  Error codes will be displayed on the bottom of the touch screen

Troubleshooting Error Codes:

1102: User Name / Password or Shared Name / Folder Name

- Try entering the domain (i.e. net/username; net@username; net\username)

- Check user permissions of network share folder

1103: Path Name or File Name

1105: SMB protocol is set to “off” (check the scanner menu on the web config.)

2101: Cannot connect

- Check network connection

- Check network host name / IP address (try IP Address instead of host name)

- If using host name check DNS settings under “Settings” -> “Advanced” -> “TCP/IP” -> “General”

2103: Timeout

2201: Network Transfer Error